Registers, Cache Memory, and Buses - Notes

What are Registers?

Registers are small storage units inside the CPU that temporarily hold data, addresses, and instructions. They allow the CPU to access information quickly and efficiently.

Key Features of Registers:

✅ Located inside the processor for fast access.
✅ Store binary data (0s and 1s).
✅ Work with D flip-flops (a group of flip-flops forms a multi-bit register).
✅ Data in registers is updated with every clock pulse.

Types of CPU Registers and Their Functions

1. Data Registers (Hold data for processing)

🔹 Accumulator (ACC): Stores numbers for arithmetic and logical operations.
🔹 General-Purpose Registers (GPRs): Can store numbers, addresses, or any data.

2. Address Registers (Hold memory locations)

🔹 Memory Address Register (MAR): Stores the address of data or instructions.
🔹 Index Registers (IX, IY): Helps in locating memory addresses by adding an offset to a base address.

3. Control Registers (Manage CPU operations)

🔹 Program Counter (PC): Points to the next instruction to be executed.
🔹 Instruction Register (IR): Holds the current instruction being processed.
🔹 Stack Pointer (SP): Keeps track of the top of the stack (used in function calls and memory management).

4. Status Registers (Track results of operations)

🔹 Flags Register: Stores flags that show outcomes of operations, like:

- Zero flag (Z): Set if the result is zero.

- Carry flag (C): Set if there is a carry/borrow in arithmetic.

- Overflow flag (V): Set if a number is too large to fit in the register.

5. Floating-Point Registers (Handle decimal numbers)

🔹 Used for floating-point arithmetic, storing real numbers (fractions, exponents).
🔹 Efficiently processes scientific and mathematical calculations.

Purpose of Registers in a CPU

✅ Fast Data Access: Registers are the fastest memory in a computer.
✅ Instruction Execution: Store instructions and data for processing.
✅ Control & Status Tracking: Maintain CPU execution status (PC, Flags).
✅ Data Storage: Temporarily hold data from RAM or input devices.
✅ Addressing: Helps in locating memory addresses for reading/writing data.

Cache Memory

🟢 Definition: A small, high-speed memory located inside or near the CPU.
🟢 Purpose: Stores frequently used instructions and data to reduce processing time.
🟢 Benefit: Faster access than RAM, reducing CPU wait time.

Buses in a Computer

Buses are communication pathways that transfer data and control signals between the CPU, memory, and input/output (I/O) devices.

Types of Buses:

🟡 Data Bus: Carries actual data between CPU, memory, and I/O devices.
🟡 Address Bus: Transfers memory addresses that the CPU needs to access.
🟡 Control Bus: Sends control signals (e.g., read/write) to manage CPU operations.

Addressing Modes - Simplified Notes

What is Addressing Mode?

Addressing mode is the method used by the CPU to access data stored in registers or memory. It defines how the operand (data) is located during program execution.

Why Do We Need Addressing Modes?

✅ Helps in efficient memory usage.
✅ Reduces the number of bits required in an instruction.
✅ Supports looping, indexing, and memory access techniques.
✅ Allows flexibility in programming.

Types of Addressing Modes

1. Implied Addressing Mode

🔹 The operand is automatically assumed and does not need to be specified in the instruction.
🔹 Example: COM A (Complement Accumulator) → The accumulator is implied as the operand.

2. Immediate Addressing Mode

🔹 The operand is directly given in the instruction itself.
🔹 Example: MOV A, #10 → The number 10 is the operand.

✔️ Advantage: Fast execution because no memory access is required.
❌ Disadvantage: The size of the operand is limited.

3. Register Addressing Mode (Register Direct Mode)

🔹 The operand is stored in a CPU register.
🔹 Example: ADD A, B → The instruction adds the contents of register B to register A.

✔️ Advantage: Fast execution as data is directly in registers.
❌ Disadvantage: Limited number of registers.

4. Register Indirect Addressing Mode

🔹 The register contains the memory address of the operand, not the operand itself.
🔹 Example: MOV A, @R0 → The address stored in R0 is used to fetch data from memory.

✔️ Advantage: Saves memory space.
❌ Disadvantage: Requires extra memory access.

5. Direct Addressing Mode

🔹 The instruction contains the actual memory address of the operand.
🔹 Example: MOV A, 5000H → The operand is stored at memory address 5000H.

✔️ Advantage: Simple and easy to understand.
❌ Disadvantage: Memory addresses are fixed, reducing flexibility.

6. Indirect Addressing Mode

🔹 The instruction contains the address of a memory location that holds the actual address of the operand.
🔹 Example: MOV A, (5000H) → The value at address 5000H is another address where the operand is stored.

✔️ Advantage: Good for handling large memory spaces.
❌ Disadvantage: Requires extra memory access, making it slower.

7. Indexed Addressing Mode

🔹 The effective address is calculated as:
EA = Address part of instruction + Content of index register
🔹 Used for accessing arrays in memory.

✔️ Advantage: Useful for looping and table lookups.
❌ Disadvantage: Extra calculation step increases processing time.

8. Auto-Increment & Auto-Decrement Addressing Mode

🔹 Similar to register indirect mode but increments or decrements the register automatically before or after accessing memory.
🔹 Example:

- MOV A, @R0+ (Auto-Increment) → Fetch operand from R0, then increase R0.

- MOV A, @R0- (Auto-Decrement) → Decrease R0, then fetch operand.

✔️ Advantage: Efficient for working with stacks and arrays.
❌ Disadvantage: Limited to specific instructions.

9. Relative Addressing Mode

🔹 The effective address is calculated using:
EA = Displacement + Program Counter (PC) 🔹 Mostly used in branching and looping instructions.

✔️ Advantage: Good for position-independent code.
❌ Disadvantage: Limited address range.

Summary Table of Addressing Modes

| Addressing Mode | How It Works | Example |
| --- | --- | --- |
| Implied | Operand is built-in | COM A |
| Immediate | Operand is in instruction | MOV A, #10 |
| Register Direct | Operand is in a register | ADD A, B |
| Register Indirect | Register holds address | MOV A, @R0 |
| Direct | Instruction has memory address | MOV A, 5000H |
| Indirect | Memory address holds another address | MOV A, (5000H) |
| Indexed | Effective address = Index + Address | MOV A, X(R1) |
| Auto-Increment/Decrement | Register auto-increments or decrements | MOV A, @R0+ |
| Relative | Effective address = PC + Displacement | JMP X(PC) |

These addressing modes help in efficient memory usage and better program execution. 🚀

Instruction Set - Easy Notes

What is an Instruction Set?

An Instruction Set is a collection of commands that a CPU can execute. These commands help the computer perform different tasks like arithmetic operations, data movement, and program control.

Types of Instructions in an Instruction Set

The instruction set of a CPU includes different types of instructions. These are classified into the following categories:

1. Data Transfer Instructions

These instructions move data from one place to another.

- MOV – Transfers data from one register/memory location to another.

- LOAD (LD) – Loads data from memory into a register.

- STORE (ST) – Stores data from a register to memory.

- PUSH/POP – Push stores data onto the stack, and Pop retrieves data from the stack.

2. Arithmetic Instructions

Used for mathematical operations.

- ADD – Adds two values.

- SUB – Subtracts one value from another.

- MUL – Multiplies two values.

- DIV – Divides one value by another.

- INC/DEC – Increments or decrements a value by 1.

3. Logical Instructions

Used for logical operations like AND, OR, XOR, etc.

- AND – Performs a bitwise AND operation.

- OR – Performs a bitwise OR operation.

- XOR – Performs an exclusive OR operation.

- NOT – Inverts bits.

- CMP – Compares two values.

4. Branching (Control Transfer) Instructions

These instructions change the sequence of execution.

- JMP (Jump) – Moves execution to a different instruction address.

- CALL/RET – CALL moves to a subroutine, RET returns to the main program.

- Conditional Jumps (JZ, JNZ, JC, JNC) – Jumps based on conditions (zero, non-zero, carry, no carry, etc.).

5. Input/Output Instructions

Used for communication with external devices.

- IN – Reads data from an input device.

- OUT – Sends data to an output device.

6. Special Instructions

These instructions control CPU operations.

- NOP (No Operation) – CPU does nothing for one cycle.

- HLT (Halt) – Stops program execution.

- WAIT – Pauses execution until an event occurs.

How the CPU Executes Instructions?

The CPU follows these steps to execute instructions:

- Fetch – Gets the instruction from memory.

- Decode – Understands what operation to perform.

- Execute – Carries out the instruction.

- Store – Saves the result if needed.

………………………………………………………………………………………………………………………………………………………….

What is a Bus?

- A bus is a set of wires used to transfer information between components in a computer system, such as the CPU, memory, and input/output devices. It helps in connecting various registers, memory units, and peripherals efficiently.

Types of Buses:

- Address Bus

  - Purpose: Carries the address of the location to be accessed in memory or an input/output device.

  - Direction: Unidirectional (from the CPU to memory or input/output devices).

  - Length of Address Bus:

    - 8085 microprocessor: 16 bits (can address 65,536 memory locations = 64KB).

    - 8086 microprocessor: 20 bits (can address 1,048,576 memory locations = 1MB).

- Data Bus

  - Purpose: Carries the actual data being transferred between the CPU and memory or input/output devices.

  - Direction: Bidirectional (data flows in both directions).

  - Example of Operation:

    - Fetch: CPU places the address on the address bus, then memory places data on the data bus, which is read by the CPU.

    - Write: CPU places the address on the address bus and data on the data bus, which memory stores at the given address.

  - Length of Data Bus:

    - 8085 microprocessor: 8 bits.

    - 8086 microprocessor: 16 bits.

- Control Bus

  - Purpose: Carries control signals from the CPU to other devices (e.g., read/write instructions, synchronization signals).

  - Lines in Control Bus:

    - Read (R') – Signals that data is being read from a device.

    - Write (W') – Signals that data is being written to a device.

    - ACK (Acknowledgement) – Signals that the device has received the data.

    - Bus Request – Signals that a device is requesting to access the bus.

    - Reset – Resets the system.

    - Clock Signals – Synchronizes the CPU with other devices.

  - Direction: Bidirectional (signals can flow both ways).

Common Bus System:

- Registers (like DR, AC, IR, and TR) are connected to the common bus.

- Registers and memory can either send or receive information from the bus, based on control signals.

- Control Signals: Help manage data transfer operations like loading (LD), incrementing (INR), or clearing (CLR) data.

Components and Functions:

- Registers:

  - Registers like AC, DR, IR, and TR hold data and can communicate with the common bus.

  - INPR (Input Register) and OUTR (Output Register) handle data between the CPU and input/output devices.

- Adder and Logic Circuit:

  - Adder & Logic Circuit receives inputs from different registers, processes data (e.g., arithmetic operations), and sends the result back to the AC (Accumulator) register.

  - Operations like adding, incrementing, or transferring data can be done in one clock cycle.

Advantages of Using Buses:

- Easy Expansion: New components can be added to the bus system without much complexity.

- Cost-Effective: Using a bus reduces the need for numerous direct connections, saving both cost and space.

- Reduced Complexity: Instead of direct connections between every pair of components, buses provide a simpler way to transfer data.

Disadvantages of Using Buses:

- Communication Bottleneck: As the number of devices increases, data transfer can become slower due to limited bandwidth.

- Speed Limitation: The maximum speed is affected by:

  - Bus Length: Longer buses can cause slower data transfer.

  - Number of Devices: More devices on the bus can slow down communication.