Control Unit

What is a Control Unit?

- The control unit is a part of the CPU responsible for ensuring that instructions are executed in the correct order.

- To achieve this, it generates control signals in the right sequence.

Types of Control Units

There are two main ways to design a control unit:

- Hardwired Control Unit

- Microprogrammed Control Unit

Hardwired Control Unit

What is a Hardwired Control Unit?

- A hardwired control unit uses specially built hardware logic circuits to generate control signals.

- The control signals are "wired in" to the hardware.

- It uses Finite State Machines (FSM) to produce these signals.

- Modifying the way signals are produced requires physically altering the circuit.

Components of a Hardwired Control Unit

A hardwired control unit consists of:

- Two decoders

- A sequence counter

- Logic gates

- Instruction Register (IR) – Holds the instruction fetched from memory.

  - The IR contains the operation code (opcode) and other bits.

  - A 3×8 decoder is used to decode part of the opcode (bits 12-14), producing outputs labeled D0 to D7.

  - Another bit of the opcode (bit 15) is sent to a flip-flop called 'I'.

- Control logic gates process the remaining opcode bits (0-11).

- Sequence Counter (SC) keeps track of instruction execution steps (counts from 0 to 15).

How a Hardwired Control Unit Works

- The operation code of an instruction is the primary input for generating control signals.

- The instruction decoder decodes the operation code.

- Only some output lines of the decoder are active at any time.

- These active lines are processed in a matrix that generates control signals for various components.

- The matrix combines:

  - Decoded instruction signals

  - Sequence control unit states

  - External signals (e.g., interrupts)

- The control unit transitions through different internal states during instruction execution.

- Upon receiving a new instruction:

  - The control unit enters the fetch state.

  - The decoded instruction determines the next execution state.

  - The state continues as long as timing signals and flags remain unchanged.

  - If any signal changes, the control unit shifts to the next state.

- Interrupts force the control unit into special states to handle them.

- The control unit uses flags and state variables to determine the correct states for executing an instruction.

- The final states involve:

  - Fetching the next instruction.

  - Sending the program counter (PC) value to memory.

  - Reading the instruction.

- If a STOP instruction is encountered, the control unit may enter an OS state and wait.

Methods for Designing Hardwired Control Logic

- Sequence Counter Method – A practical way to design complex controllers.

- Delay Element Method – Uses timed delays to generate control signals.

- State Table Method – A formal approach using state tables.

Advantages of Hardwired Control Units

- Faster execution due to combinational circuits.

- Signal generation delay depends on the number of gates, which can be small.

- Optimized for the fastest operation.

- Generally faster than microprogrammed control units.

Disadvantages of Hardwired Control Units

- Complex design when more control signals are needed.

- Difficult to modify control signals, as rewiring the hardware is required.

- Adding new features is time-consuming and challenging.

- Troubleshooting and debugging initial designs is difficult.

- Higher cost due to hardware complexity.

Micro-programmed Control Unit

What is a Micro-programmed Control Unit?

- It functions like a program that instructs the computer’s control unit on what to do.

- This program consists of small instructions called microinstructions.

- These microinstructions direct the computer to execute a series of micro-operations.

How It Works

- The control unit has a special control memory (such as ROM or RAM) that stores microprograms, also called the control store.

- When the computer executes an instruction, the control unit retrieves the corresponding microprogram from the control store.

- The operation code (opcode) helps locate the starting address of the required microprogram.

- The control unit reads microinstructions sequentially from the control store.

- Each microinstruction contains control signals that direct various parts of the computer.

- A control word is a collection of control signals within a microinstruction.

- The microinstruction also specifies where to find the next microinstruction, managed by the next address generator (also known as the microprogram sequencer).

Key Terms

- Control Word – A set of bits representing control signals.

- Micro-routine – A sequence of control words for one machine instruction.

- Micro-instruction – A single control word in a micro-routine.

- Micro-program – A sequence of micro-instructions stored in control memory.

- Control Store – A specialized memory that holds micro-routines for all instructions.

Micro-programmed vs. Hardwired Control Unit

| Feature | Micro-programmed Control Unit | Hardwired Control Unit |
| --- | --- | --- |
| Control Mechanism | Uses a control store with encoded control signals | Uses fixed logic circuits |
| Design Approach | Software-based | Hardware-based |
| Modification | Easier to modify and update | Difficult to modify |
| Speed | Slower due to memory access | Faster due to direct signal generation |
| Complexity | More systematic and structured | More complex as circuits grow |
| Flexibility | High flexibility (easier to add new instructions) | Less flexible (modifications require hardware changes) |
| Use Case | Preferred for complex systems and adaptable processors | Used in high-speed applications |

A microprogram-controlled computer's control unit is sometimes called a "computer within a computer".

Types of Micro-programmed Control Units

1. Horizontal Micro-programmed Control Unit

- Each control signal has its own bit in the microinstruction (1 bit per control signal).

- Requires longer control words.

- Supports high parallelism, allowing multiple control signals to be active simultaneously.

- Faster, as no extra decoding hardware is required.

- More flexible and well-suited for parallel processing.

2. Vertical Micro-programmed Control Unit

- Control signals are encoded, requiring fewer bits (Log₂(Number of control signals)).

- Uses shorter control words.

- Supports a lower degree of parallelism (usually 0 or 1 control signal active at a time).

- Slower, since decoders are needed to generate actual control signals.

- Easier to modify than hardwired control but less flexible than horizontal microprogramming.

Advantages of Micro-programmed Control Units

✔ Systematic and easier to design.
✔ Simpler troubleshooting and modifications.
✔ Organized control structure.
✔ Lower design complexity, potentially reducing cost and errors.
✔ More adaptable to changes and updates.
✔ Handles complex functions efficiently.
✔ Enables software-based control operations.

Disadvantages of Micro-programmed Control Units

❌ Slower than hardwired control units.
❌ Higher initial cost due to control memory and additional circuitr

Comparison Between Hardwired and Micro-programmed Control Units

Control Units: Two Main Types

A CPU can control its operations using two main approaches:

- Hardwired Control Unit

- Micro-programmed Control Unit

1. Hardwired Control Unit

- Functions as a fixed electronic circuit that generates control signals for instruction execution.

- Uses hardware circuits to generate control signals.

- Implements logic circuits such as a state counter and Programmable Logic Array (PLA).

Key Features

- Speed: Faster than micro-programmed control units.

- Modification: Difficult to modify since changes require circuit redesign.

- Handling Complex Instructions: Struggles with complex instructions as circuits become overly complicated.

- Cost: More expensive due to extensive use of logic gates.

- Number of Instructions: Supports a limited number of instructions.

- Instruction Decoding: More difficult to perform.

- Control Memory: Does not use control memory.

- Flexibility: Less flexible due to fixed hardware design.

- Applications: Commonly used in Reduced Instruction Set Computers (RISC).

2. Micro-programmed Control Unit

- Uses a programming approach with microinstructions to generate control signals.

- Control signals are produced by executing microinstructions stored in control memory.

Key Features

- Speed: Slower than hardwired units due to frequent memory access.

- Modification: Easy to modify by changing microinstructions in control memory.

- Handling Complex Instructions: Can easily handle complex instructions.

- Cost: Less costly since it relies on microinstructions instead of hardware circuits.

- Number of Instructions: Can support many instructions.

- Instruction Decoding: Easier to perform.

- Control Memory: Uses control memory to store microinstructions.

- Flexibility: More flexible since modifications are software-based.

- Applications: Commonly used in Complex Instruction Set Computers (CISC).

Main Differences: Hardwired vs. Micro-programmed Control Units

| Feature | Hardwired Control Unit | Micro-programmed Control Unit |
| --- | --- | --- |
| Implementation | Hardware circuit / Circuitry approach | Programming |
| Control Signal Generation | Logic circuit / Hardwired form | Microinstructions / Control memory |
| Modification | Very difficult (requires circuit redesign) | Very easy (change microinstructions) |
| Handling Complex Instructions | Difficult to handle | Easily handles complex instructions |
| Cost | More costly (uses logic gates) | Less costly (uses microinstructions) |
| Number of Instructions | Limited | Many |
| Speed | Faster | Slower (due to memory accesses) |
| Instruction Set | Small (RISC) | Large (CISC) |
| Control Memory | No | Yes |