Memory HierarchyS

The memory hierarchy is a structured organization of different types of memory in a computer, arranged in a pyramid-like fashion. It helps manage computer storage based on speed and capacity.

Levels of the Memory Hierarchy

The memory hierarchy can be visualized as different shelves in a storage system, with the fastest but smallest shelves at the top and the slowest but largest at the bottom.

- Level 0: CPU Registers

  - Description: Tiny, super-fast storage spaces inside the CPU.

  - Characteristics:

    - Fastest access time.

    - Most expensive type of memory.

    - Smallest size, typically a few kilobytes (KB).

    - Made using flip-flops.

- Level 1: Cache Memory

  - Description: Slightly larger and slower than registers, but still very fast. Stores frequently used pieces of programs.

  - Characteristics:

    - Expensive but larger than registers, typically measured in megabytes (MB).

    - Made using static RAM (SRAM).

- Level 2: Main Memory (Primary Memory)

  - Description: The main working memory of the computer, directly interacting with the CPU and slower storage devices.

  - Characteristics:

    - Less expensive than cache, larger, usually measured in gigabytes (GB).

    - Made using dynamic RAM (DRAM).

- Level 3: Magnetic Disks (Secondary Storage)

  - Description: Used for long-term storage and backup.

  - Characteristics:

    - Cheaper and much larger than main memory, often a few terabytes (TB).

- Level 4: Optical Disks or Magnetic Tapes (Tertiary Memory)

  - Description: Used for removable file storage, cheapest and largest.

  - Characteristics:

    - Ranges from 1 to 20 TB.

Diagram of the Memory Hierarchy

Copy

^ Fastest, Most Expensive, Smallest Capacity

|

-------------------------

|       CPU Registers (Level 0)       |

-------------------------

|

-------------------------

|     Cache Memory (Level 1)      |

-------------------------

|

-------------------------

|   Main Memory (Level 2)    |

-------------------------

|

-------------------------

|  Magnetic Disks (Level 3)   |

-------------------------

|

-------------------------

| Tertiary Memory (Level 4) |

-------------------------

|

v Slowest, Least Expensive, Largest Capacity

Key Characteristics of the Memory Hierarchy

- Capacity increases: The amount of data that can be stored gets larger as you move down the hierarchy.

- Access time increases: It takes longer to retrieve data from lower levels.

- Cost per bit decreases: Each unit of storage becomes cheaper.

- Speed decreases: The speed at which data can be read or written becomes slower.

Internal vs. External Memory

- Internal (Primary) Memory: Includes registers, cache, and main memory. Directly accessed by the processor.

- External (Secondary) Memory: Includes magnetic disks, optical disks, and magnetic tapes. Accessed via input/output operations.

Why Use a Memory Hierarchy?

The primary reason is to speed up the computer by bridging the speed gap between the fast CPU and slower main memory. This is achieved by keeping frequently used data in faster, smaller memories closer to the CPU, leveraging the principles of locality of reference:

- Temporal Locality: Data used recently is likely to be used again soon.

- Spatial Locality: Data near recently used data is likely to be used soon.

Main Memory

Main memory (or primary memory) is the central storage of a computer where programs and data are kept while in use. It needs to be relatively fast and large, made using semiconductor integrated circuits.

Types of Main Memory Chips

- RAM (Random Access Memory)

  - Static RAM (SRAM):

    - Stores data using flip-flops.

    - Faster and easier to use but more expensive and has smaller storage capacity.

    - Volatile (data stays valid as long as power is on).

  - Dynamic RAM (DRAM):

    - Stores data as electrical charges in capacitors.

    - Slower but cheaper and can store more data.

    - Requires periodic refreshing (volatile).

- ROM (Read Only Memory)

  - Used to store permanent programs and data (non-volatile).

  - Stores the bootstrap loader, which starts the computer.

  - Read-only in normal operation.

Memory Address Map

A table showing which address ranges are assigned to each memory chip (RAM or ROM) in the system, helping the computer locate different parts of its memory.

Memory Connection to the CPU

- RAM and ROM chips are connected to the CPU using data and address buses.

- Address bus lines select specific locations within a chip.

- Data bus allows data transfer between memory and CPU.

- Control signals (read/write) from the CPU tell the memory what operation to perform.

Auxiliary Memory

Auxiliary memory (secondary or external memory) is used for long-term storage of data and programs, differing from primary memory (RAM) which is for short-term storage.

Key Characteristics of Auxiliary Memory

- Larger Capacity: Can store much more data than primary memory.

- Slower Speed: Accessing data is slower than from RAM.

- Non-Volatile: Data remains stored even when the computer is off.

How Auxiliary Memory Works

The CPU cannot directly access secondary memory. Data from secondary storage is first loaded into RAM and then sent to the CPU.

Examples of Secondary Memory Devices

- Hard Disk Drives (HDDs): Traditional mechanical drives.

- Solid-State Drives (SSDs): Faster and more durable than HDDs.

- Magnetic Tapes: Older technology for large-capacity backup.

- Optical Discs: CDs, DVDs, and Blu-ray discs.

- Flash Memory: USB drives and memory cards.

Fixed vs. Removable Storage

- Fixed Storage: Internal devices like HDDs and SSDs.

- Removable Storage: External devices like USB drives, CDs, DVDs, and memory cards.

Advantages of Secondary Storage

- Permanent Storage: Keeps data safe when power is off.

- Portability: Easy data transfer.

- Large Capacity: Can store vast amounts of information.

- Cost-Effective: Cheaper per unit of storage than primary memory.

Disadvantages of Secondary Storage

- Slower Access Times: Takes longer to retrieve data.

- Mechanical Failures: HDDs can break down.

- Limited Lifespan: Some devices have read/write cycle limits.

- Data Corruption: Data can be damaged.

Associative Memory

Associative memory (Content Addressable Memory or CAM) is a special type of memory where data is accessed based on its content rather than its address.

How it Works

Instead of providing a memory location, you provide a piece of data (the argument), and the associative memory searches all its stored words in parallel to find a match.

Applications of Associative Memory

- Memory Allocation.

- Database Management Systems.

- Networking.

- Image Processing.

- Artificial Intelligence.

- Page Tables (Virtual Memory).

- Neural Networks.

Advantages of Associative Memory

- Fast Search Time: Parallel search is very quick.

- Suitable for Parallel Searches.

- Speeds up Databases.

Disadvantages of Associative Memory

- More Expensive than RAM.

- Complex Cells: Each memory cell needs storage and logic circuits.

Cache Memory

Cache memory is a small, fast memory that stores copies of frequently used data from main memory.

Need for Cache Memory

Cache memory minimizes the time gap between the CPU and main memory, reducing CPU wait time and improving performance.

Levels of Cache Memory

- L1 Cache (Level 1): Fastest and smallest, located inside each CPU core.

- L2 Cache (Level 2): Can be inside or outside the CPU core, shared between cores.

- L3 Cache (Level 3): Located outside the CPU, shared by all cores.

Locality of Reference

Cache memory leverages locality of reference:

- Temporal Locality: Data recently used is likely to be used again.

- Spatial Locality: Data near recently used data is likely to be used soon.

Principles of Cache

- Cache Hit: Data found in cache, accessed quickly.

- Cache Miss: Data not found in cache, fetched from main memory.

Cache Performance

- Hit Ratio: Percentage of times data is found in cache.

Cache Mapping

- Direct Mapping: Each block in main memory maps to one specific line in cache.

- Fully Associative Mapping: A block can be placed in any available line in cache.

- K-way Set Associative Mapping: Blocks map to any line within a specific set.

Cache Replacement Policies :

Cache memory is small, so when it gets full, the system must remove old data to make space for new data. The rules that decide which data to remove are called Cache Replacement Policies.

FIFO (First In First Out): Replaces the oldest block.

- LRU (Least Recently Used): Replaces the least recently used block.

- LFU (Least Frequently Used): Replaces the least frequently used block.

- Random Replacement (RR): Randomly selects a block for replacement.

Cache Design Issues

- Cache Addresses: Logical (Virtual) vs. Physical Cache.

- Cache Size: Larger caches improve hit ratio but increase cost and access time.

- Replacement Algorithm: Affects hit ratio.

- Write Policy: Write Through vs. Write Back.

- Line Size (Block Size): Affects hit ratio and cache efficiency.

- Number of Caches (Multilevel Caches): Different trade-offs between speed and size.

- Mapping Function: Direct, associative, set-associative mapping.

Cache vs. RAM

| Feature | RAM | Cache |
| --- | --- | --- |
| Size | Larger (MB to GB) | Smaller (KB to a few MB) |
| Purpose | Stores currently processed data | Holds frequently accessed data |
| Interaction | OS manages data transfer from secondary memory | OS manages data transfer from primary memory |
| Misses | RAM miss never occurs before CPU access | Cache miss can occur if data not found |
| Access by | Address | Address |
| Cost | Less expensive than cache | More expensive than RAM |

Differences between Associative and Cache Memory

| Feature | Associative Memory | Cache Memory |
| --- | --- | --- |
| Access By | Content | Address |
| Goal | Reduce time to find item in memory | Reduce average memory access time |
| Basic Characteristic | Logic circuit for matching content | Fast access |
| Cost | Less expensive than cache memory | More expensive than associative memory |
| Search Mechanism | Suitable for parallel data search | Useful in increasing efficiency of data retrieval |

Virtual Memory, Paging, and Segmentation

Virtual Memory

Virtual memory is a technique that makes the computer think it has more RAM than it actually does by using a portion of the hard disk or SSD as an extension of RAM. Managed by the Memory Management Unit (MMU).

Key Concepts

- Page: A fixed-size block of virtual memory on the disk.

- Frame: A fixed-size block of physical memory (RAM).

- Virtual Address: Address generated by the CPU.

- Physical Address: Actual address in RAM.

- Page Table: Maps virtual pages to physical frames.

- Translation Lookaside Buffer (TLB): Caches recent translations of virtual to physical addresses.

Paging

Paging divides physical memory into fixed-size frames and virtual memory into fixed-size pages. Pages are loaded into frames as needed.

Paging Process

- CPU generates a virtual address.

- Page number indexes the page table.

- Page table entry provides the base address of the frame.

- Page offset is added to the base address to get the physical address.

- MMU performs the translation, with TLB caching recent translations.

Advantages of Paging

- Avoids external fragmentation.

- Frames don't need to be contiguous.

- Transparent to the programmer.

- Avoids internal fragmentation on modern systems.

Disadvantages of Paging

- Could cause internal fragmentation on older systems.

- Memory lookup might take longer due to page table lookup.

Segmentation

Segmentation divides virtual memory into variable-sized blocks called segments, each corresponding to a logical part of a program.

Segmentation Process

- CPU generates a logical address.

- Segment number indexes the segment table.

- Segment table entry contains the base address and limit of the segment.

- Segment offset is checked against the limit and added to the base address to get the physical address.

Advantages of Segmentation

- No internal fragmentation.

- Smaller segment tables.

- Easier to manage logically related parts of a program.

Disadvantages of Segmentation

- Suffers from external fragmentation.

- Requires programmer intervention.

- Limited support in modern systems.

Segmented Paging

Combines paging and segmentation, dividing memory into variable-sized segments, each further divided into fixed-size pages.

Key Differences: Paging and Segmentation

| Feature | Paging | Segmentation |
| --- | --- | --- |
| Size | Fixed-size pages and frames | Variable-size segments |
| Fragmentation | Internal (older systems), None (modern) | External |
| Tables | Page tables (slower lookup, sped up by TLB) | Segment tables (faster lookup) |
| Availability | Widely available | Limited in modern systems |

Memory Management Hardware

Memory management hardware is crucial for efficiently managing and allocating memory resources.

Components of Memory Management Hardware

- Memory Management Unit (MMU): Translates virtual addresses to physical addresses.

- Translation Lookaside Buffer (TLB): Caches recent address translations.

- Memory Segmentation Unit: Divides memory into fixed-size segments.

- Memory Protection Unit (MPU): Enforces access permissions.

Functions of Memory Management Hardware

- Address Translation: Converts virtual addresses to physical addresses.

- Memory Allocation: Assigns and deallocates memory resources.

- Memory Protection: Enforces access rights.

- Virtual Memory Management: Manages mapping between virtual and physical addresses.

Techniques Used by Memory Management Hardware

- Paging: Divides memory into fixed-size pages and frames.

- Segmentation: Divides memory into logical segments.

- Virtual Memory: Uses disk storage to extend RAM.

- Memory Protection: Implements access permissions and privilege levels.

Summary

Memory management hardware, particularly the MMU and TLB, is essential for the efficient and secure operation of modern computer systems by managing memory access, allocation, and the implementation of virtual memory techniques.